University of Asia Pacific

Department of Computer Science and Engineering

Course Code : CSE 450 (VLSI Lab)

Final Project | Spring 2026

| Total Marks | 50 ( Viva → 30, Report & VrC → 10, Presentation → 10 ) |  |  |
| --- | --- | --- | --- |
| Student Name |  | ID |  |

Final Project: Mini 8-bit Accumulator CPU

Objective. Design and implement a small single-cycle 8-bit accumulator CPU in VHDL. The CPU supports only five instructions: two are mandatory for every student, and the remaining three are assigned to you according to your Student ID (see the “Instruction Set Assignment” page: the last digit of your Student ID decides your set). You must also verify the design with a testbench that runs the program given for your set in Appendix B.

1. System Architecture

The CPU is built from the blocks shown below. Every sequential block (PC, ACC, R, Z) is clocked on the rising edge of CLK and cleared by a synchronous, active-high RESET.

| Block | Description |
| --- | --- |
| PC | 4-bit program counter. Holds the address of the instruction being executed. After every instruction it becomes PC + 1, except when JZ is taken. |
| Program ROM | 16 locations x 12 bits, written as a constant array in VHDL. It is read combinationally using PC (no clock needed). Unused locations contain 0x000 (NOP). |
| Control Unit | Decodes opcode[11:8] and generates the ALU function select and the load/enable signals for ACC, R, Z and PC. |
| ALU | 8-bit combinational unit working on ACC and the 8-bit operand. Only the operations of your own instruction set are required. |
| ACC, R, Z | ACC: 8-bit accumulator, also the CPU output Q[7:0]. R: 8-bit holding register (only if your set has STA / LDR). Z: 1-bit flag, 1 when the value written to ACC is 0x00. |

2. Instruction Format

Each instruction is 12 bits: instr[11:8] = opcode (4 bits) and instr[7:0] = operand (8-bit immediate). Every instruction takes exactly one clock cycle (fetch, decode and execute happen in the same cycle, the result is written at the rising edge).

Mandatory instructions (all students):

| Mnemonic | Opcode | Operation | Flags |
| --- | --- | --- | --- |
| LDI imm | 0001 | ACC ← imm | Z updated |
| ADD imm | 0010 | ACC ← ACC + imm (8-bit, carry discarded) | Z updated |

Instruction pool (you implement only the three assigned to you):

| Mnemonic | Opcode | Operation | Flags |
| --- | --- | --- | --- |
| SUB imm | 0011 | ACC ← ACC − imm (modulo 256) | Z updated |
| AND imm | 0100 | ACC ← ACC AND imm | Z updated |
| OR imm | 0101 | ACC ← ACC OR imm | Z updated |
| XOR imm | 0110 | ACC ← ACC XOR imm | Z updated |
| NOT | 0111 | ACC ← NOT ACC (operand ignored) | Z updated |
| SHL | 1000 | ACC ← ACC shifted left by 1, 0 enters LSB (operand ignored) | Z updated |
| SHR | 1001 | ACC ← ACC shifted right by 1, 0 enters MSB (operand ignored) | Z updated |
| STA | 1010 | R ← ACC (operand ignored, ACC unchanged) | Z unchanged |
| LDR | 1011 | ACC ← R (operand ignored) | Z updated |
| JZ addr | 1100 | If Z = 1 then PC ← operand[3:0], else PC ← PC + 1 | Z unchanged |

Note: if your set contains LDR it also contains STA. Any opcode not in your set (including 0000) behaves as NOP: PC ← PC + 1 and nothing else changes.

3. Reset Behaviour

RESET = 1 at a rising edge: PC ← 0, ACC ← 0x00, R ← 0x00, Z ← 1 (RESET has priority over everything). RESET = 0 at a rising edge: execute the instruction at ROM[PC]. Between clock edges all registers keep their values.

4. Implementation Requirements

- Use VHDL with IEEE.STD_LOGIC_1164.ALL (IEEE.NUMERIC_STD is allowed for arithmetic).

- Modular design: separate entities for PC, Program ROM, Control Unit, ALU and ACC/R/Z, connected in a top-level CPU entity.

- RESET must be synchronous and active-high. All state changes must occur on the rising edge of CLK.

- The top level must expose CLK, RESET, Q[7:0] (the ACC), the Z flag and, for checking, PC[3:0].

- Implement only the five instructions of your own set. The ROM must hold exactly the Appendix B program of your set; the testbench must generate a clock, apply RESET for one rising edge, and run until the last instruction has executed.

Instruction Set Assignment

How to find your set: take the last digit of your Student ID (ID mod 10). That digit is your set number in the table below. Example: ID 22201014 → last digit 4 → Set 4.

Every student implements the two mandatory instructions LDI and ADD plus the three assigned instructions of his/her set. Total = 5 instructions.

| Last digit of ID | Set | Mandatory | Assigned | Complete instruction set of the CPU |
| --- | --- | --- | --- | --- |
| 0 | Set 0 | LDI, ADD | AND, JZ, NOT | LDI, ADD, AND, JZ, NOT |
| 1 | Set 1 | LDI, ADD | SUB, AND, OR | LDI, ADD, SUB, AND, OR |
| 2 | Set 2 | LDI, ADD | XOR, NOT, SHL | LDI, ADD, XOR, NOT, SHL |
| 3 | Set 3 | LDI, ADD | SHR, SUB, XOR | LDI, ADD, SHR, SUB, XOR |
| 4 | Set 4 | LDI, ADD | STA, LDR, AND | LDI, ADD, STA, LDR, AND |
| 5 | Set 5 | LDI, ADD | OR, NOT, SHR | LDI, ADD, OR, NOT, SHR |
| 6 | Set 6 | LDI, ADD | SUB, JZ, SHL | LDI, ADD, SUB, JZ, SHL |
| 7 | Set 7 | LDI, ADD | AND, XOR, SHR | LDI, ADD, AND, XOR, SHR |
| 8 | Set 8 | LDI, ADD | STA, LDR, SUB | LDI, ADD, STA, LDR, SUB |
| 9 | Set 9 | LDI, ADD | NOT, SHL, JZ | LDI, ADD, NOT, SHL, JZ |

Appendix B: Required Test Program and Expected Results

Store the program of your set (Appendix B tables are ordered by set number 0–9) in the Program ROM. Apply RESET for one rising edge, then run the CPU for one clock cycle per executed instruction. The ACC, Z, R and next-PC columns show the values after the corresponding rising edge. Addresses that are not listed contain 0x000 (NOP). Operands are in hexadecimal. For NOT, SHL, SHR, STA and LDR the operand field is 0x00.

ROM encoding: instr[11:0] = opcode (4 bit) & operand (8 bit). Example: LDI 0x3C = 0001 00111100 = 0x13C.

Set 0: LDI, ADD, AND, JZ, NOT

ROM contents: 0: LDI 0xA5 → 0x1A5   |   1: AND 0x5A → 0x45A   |   2: JZ 0x05 → 0xC05   |   3: ADD 0x01 → 0x201   |   4: ADD 0x01 → 0x201   |   5: NOT → 0x700   |   6: AND 0x0F → 0x40F   |   7: JZ 0x00 → 0xC00   |   8: ADD 0x01 → 0x201

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0xA5 | 0xA5 | 0 | 10100101 | 1 |
| 2 | 1 | AND 0x5A | 0x00 | 1 | 00000000 | 2 |
| 3 | 2 | JZ 0x05 (taken) | 0x00 | 1 | 00000000 | 5 |
| 4 | 5 | NOT | 0xFF | 0 | 11111111 | 6 |
| 5 | 6 | AND 0x0F | 0x0F | 0 | 00001111 | 7 |
| 6 | 7 | JZ 0x00 (not taken) | 0x0F | 0 | 00001111 | 8 |
| 7 | 8 | ADD 0x01 | 0x10 | 0 | 00010000 | 9 |

Set 1: LDI, ADD, SUB, AND, OR

ROM contents: 0: LDI 0x3C → 0x13C   |   1: ADD 0x0F → 0x20F   |   2: SUB 0x0B → 0x30B   |   3: AND 0xF0 → 0x4F0   |   4: OR 0x0A → 0x50A   |   5: SUB 0x4A → 0x34A   |   6: SUB 0x01 → 0x301

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0x3C | 0x3C | 0 | 00111100 | 1 |
| 2 | 1 | ADD 0x0F | 0x4B | 0 | 01001011 | 2 |
| 3 | 2 | SUB 0x0B | 0x40 | 0 | 01000000 | 3 |
| 4 | 3 | AND 0xF0 | 0x40 | 0 | 01000000 | 4 |
| 5 | 4 | OR 0x0A | 0x4A | 0 | 01001010 | 5 |
| 6 | 5 | SUB 0x4A | 0x00 | 1 | 00000000 | 6 |
| 7 | 6 | SUB 0x01 | 0xFF | 0 | 11111111 | 7 |

Set 2: LDI, ADD, XOR, NOT, SHL

ROM contents: 0: LDI 0x55 → 0x155   |   1: XOR 0xFF → 0x6FF   |   2: NOT → 0x700   |   3: SHL → 0x800   |   4: ADD 0x56 → 0x256   |   5: XOR 0x81 → 0x681   |   6: SHL → 0x800

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0x55 | 0x55 | 0 | 01010101 | 1 |
| 2 | 1 | XOR 0xFF | 0xAA | 0 | 10101010 | 2 |
| 3 | 2 | NOT | 0x55 | 0 | 01010101 | 3 |
| 4 | 3 | SHL | 0xAA | 0 | 10101010 | 4 |
| 5 | 4 | ADD 0x56 | 0x00 | 1 | 00000000 | 5 |
| 6 | 5 | XOR 0x81 | 0x81 | 0 | 10000001 | 6 |
| 7 | 6 | SHL | 0x02 | 0 | 00000010 | 7 |

Set 3: LDI, ADD, SHR, SUB, XOR

ROM contents: 0: LDI 0x80 → 0x180   |   1: SHR → 0x900   |   2: SUB 0x10 → 0x310   |   3: ADD 0x0F → 0x20F   |   4: XOR 0x3F → 0x63F   |   5: ADD 0x07 → 0x207   |   6: SHR → 0x900

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0x80 | 0x80 | 0 | 10000000 | 1 |
| 2 | 1 | SHR | 0x40 | 0 | 01000000 | 2 |
| 3 | 2 | SUB 0x10 | 0x30 | 0 | 00110000 | 3 |
| 4 | 3 | ADD 0x0F | 0x3F | 0 | 00111111 | 4 |
| 5 | 4 | XOR 0x3F | 0x00 | 1 | 00000000 | 5 |
| 6 | 5 | ADD 0x07 | 0x07 | 0 | 00000111 | 6 |
| 7 | 6 | SHR | 0x03 | 0 | 00000011 | 7 |

Set 4: LDI, ADD, STA, LDR, AND

ROM contents: 0: LDI 0x1E → 0x11E   |   1: STA → 0xA00   |   2: ADD 0x22 → 0x222   |   3: AND 0x60 → 0x460   |   4: LDR → 0xB00   |   5: ADD 0x01 → 0x201   |   6: AND 0x0F → 0x40F   |   7: STA → 0xA00   |   8: LDI 0x00 → 0x100   |   9: LDR → 0xB00

| Cycle | PC | Instruction executed | ACC (hex) | Z | R (hex) | ACC (bin) | Next PC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 0x00 | 00000000 | 0 |
| 1 | 0 | LDI 0x1E | 0x1E | 0 | 0x00 | 00011110 | 1 |
| 2 | 1 | STA | 0x1E | 0 | 0x1E | 00011110 | 2 |
| 3 | 2 | ADD 0x22 | 0x40 | 0 | 0x1E | 01000000 | 3 |
| 4 | 3 | AND 0x60 | 0x40 | 0 | 0x1E | 01000000 | 4 |
| 5 | 4 | LDR | 0x1E | 0 | 0x1E | 00011110 | 5 |
| 6 | 5 | ADD 0x01 | 0x1F | 0 | 0x1E | 00011111 | 6 |
| 7 | 6 | AND 0x0F | 0x0F | 0 | 0x1E | 00001111 | 7 |
| 8 | 7 | STA | 0x0F | 0 | 0x0F | 00001111 | 8 |
| 9 | 8 | LDI 0x00 | 0x00 | 1 | 0x0F | 00000000 | 9 |
| 10 | 9 | LDR | 0x0F | 0 | 0x0F | 00001111 | 10 |

Set 5: LDI, ADD, OR, NOT, SHR

ROM contents: 0: LDI 0x0F → 0x10F   |   1: OR 0xA0 → 0x5A0   |   2: NOT → 0x700   |   3: SHR → 0x900   |   4: ADD 0xD8 → 0x2D8   |   5: OR 0x81 → 0x581   |   6: SHR → 0x900

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0x0F | 0x0F | 0 | 00001111 | 1 |
| 2 | 1 | OR 0xA0 | 0xAF | 0 | 10101111 | 2 |
| 3 | 2 | NOT | 0x50 | 0 | 01010000 | 3 |
| 4 | 3 | SHR | 0x28 | 0 | 00101000 | 4 |
| 5 | 4 | ADD 0xD8 | 0x00 | 1 | 00000000 | 5 |
| 6 | 5 | OR 0x81 | 0x81 | 0 | 10000001 | 6 |
| 7 | 6 | SHR | 0x40 | 0 | 01000000 | 7 |

Set 6: LDI, ADD, SUB, JZ, SHL

ROM contents: 0: LDI 0x02 → 0x102   |   1: SUB 0x01 → 0x301   |   2: JZ 0x07 → 0xC07   |   3: SUB 0x01 → 0x301   |   4: JZ 0x07 → 0xC07   |   5: ADD 0xAA → 0x2AA   |   6: ADD 0xAA → 0x2AA   |   7: LDI 0x21 → 0x121   |   8: SHL → 0x800   |   9: ADD 0x01 → 0x201

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0x02 | 0x02 | 0 | 00000010 | 1 |
| 2 | 1 | SUB 0x01 | 0x01 | 0 | 00000001 | 2 |
| 3 | 2 | JZ 0x07 (not taken) | 0x01 | 0 | 00000001 | 3 |
| 4 | 3 | SUB 0x01 | 0x00 | 1 | 00000000 | 4 |
| 5 | 4 | JZ 0x07 (taken) | 0x00 | 1 | 00000000 | 7 |
| 6 | 7 | LDI 0x21 | 0x21 | 0 | 00100001 | 8 |
| 7 | 8 | SHL | 0x42 | 0 | 01000010 | 9 |
| 8 | 9 | ADD 0x01 | 0x43 | 0 | 01000011 | 10 |

Set 7: LDI, ADD, AND, XOR, SHR

ROM contents: 0: LDI 0xF3 → 0x1F3   |   1: AND 0x3C → 0x43C   |   2: XOR 0x11 → 0x611   |   3: SHR → 0x900   |   4: ADD 0x30 → 0x230   |   5: XOR 0x40 → 0x640

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0xF3 | 0xF3 | 0 | 11110011 | 1 |
| 2 | 1 | AND 0x3C | 0x30 | 0 | 00110000 | 2 |
| 3 | 2 | XOR 0x11 | 0x21 | 0 | 00100001 | 3 |
| 4 | 3 | SHR | 0x10 | 0 | 00010000 | 4 |
| 5 | 4 | ADD 0x30 | 0x40 | 0 | 01000000 | 5 |
| 6 | 5 | XOR 0x40 | 0x00 | 1 | 00000000 | 6 |

Set 8: LDI, ADD, STA, LDR, SUB

ROM contents: 0: LDI 0x64 → 0x164   |   1: STA → 0xA00   |   2: SUB 0x14 → 0x314   |   3: ADD 0x0A → 0x20A   |   4: STA → 0xA00   |   5: SUB 0x5A → 0x35A   |   6: LDR → 0xB00   |   7: SUB 0x60 → 0x360

| Cycle | PC | Instruction executed | ACC (hex) | Z | R (hex) | ACC (bin) | Next PC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 0x00 | 00000000 | 0 |
| 1 | 0 | LDI 0x64 | 0x64 | 0 | 0x00 | 01100100 | 1 |
| 2 | 1 | STA | 0x64 | 0 | 0x64 | 01100100 | 2 |
| 3 | 2 | SUB 0x14 | 0x50 | 0 | 0x64 | 01010000 | 3 |
| 4 | 3 | ADD 0x0A | 0x5A | 0 | 0x64 | 01011010 | 4 |
| 5 | 4 | STA | 0x5A | 0 | 0x5A | 01011010 | 5 |
| 6 | 5 | SUB 0x5A | 0x00 | 1 | 0x5A | 00000000 | 6 |
| 7 | 6 | LDR | 0x5A | 0 | 0x5A | 01011010 | 7 |
| 8 | 7 | SUB 0x60 | 0xFA | 0 | 0x5A | 11111010 | 8 |

Set 9: LDI, ADD, NOT, SHL, JZ

ROM contents: 0: LDI 0xFF → 0x1FF   |   1: NOT → 0x700   |   2: JZ 0x05 → 0xC05   |   3: ADD 0x77 → 0x277   |   4: ADD 0x77 → 0x277   |   5: ADD 0x11 → 0x211   |   6: SHL → 0x800   |   7: JZ 0x00 → 0xC00   |   8: SHL → 0x800

| Cycle | PC | Instruction executed | ACC (hex) | Z | ACC (binary) | Next PC |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | - | RESET = 1 | 0x00 | 1 | 00000000 | 0 |
| 1 | 0 | LDI 0xFF | 0xFF | 0 | 11111111 | 1 |
| 2 | 1 | NOT | 0x00 | 1 | 00000000 | 2 |
| 3 | 2 | JZ 0x05 (taken) | 0x00 | 1 | 00000000 | 5 |
| 4 | 5 | ADD 0x11 | 0x11 | 0 | 00010001 | 6 |
| 5 | 6 | SHL | 0x22 | 0 | 00100010 | 7 |
| 6 | 7 | JZ 0x00 (not taken) | 0x22 | 0 | 00100010 | 8 |
| 7 | 8 | SHL | 0x44 | 0 | 01000100 | 9 |

Submission Guidelines

Submit your VHDL source files, testbench and a project report within the 2-week period. Include this project description (question, assignment page and Appendix B) at the front of your report and start your own report on a new page after it. The report must include, but is not limited to, the following points.

- Your Student ID, assigned set number and the three assigned instructions (highlighted in yellow color)

- Attach your entity code (all modules), testbench code and output visuals (waveform with PC, ACC, Z and R if used)

- Challenges faced during the experiment, and conclusion